Problem 4: Variable Length Subnet Masking (VLSM)

Allocate subnets for the address block 192.168.1.0/24 to satisfy the following requirements, maximizing address efficiency:

​Department A (Engineering): 60 usable hosts

​Department B (Sales): 28 usable hosts

​Department C (HR): 12 usable hosts

​Department D (WAN Link): 2 usable hosts

​For each department, list:

​Allocated Network Address & Prefix Length (/XX)

​Subnet Mask

​First Usable IP

​Last Usable IP

​Broadcast Address

| 192.168.1.0/24 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| …11000000 | /26 | 64 | 62 |  |  |  |
| Department A (Engineering): | 60 usable hosts |  |  |  |  |  |
| Allocated Network Address & Prefix Length (/XX) | 192.168.1.0/26 | even |  |  |  |  |
| ​Subnet Mask | 255.255.255.192 |  |  |  |  |  |
| ​First Usable IP | 192.168.1.1 | odd |  |  |  |  |
| ​Last Usable IP | 192.168.1.62 | even |  |  |  |  |
| ​Broadcast Address | 192.168.1.63 | odd |  |  |  |  |
| 192.168.1.64/26 |  |  |  |  |  |  |
| …11100000 | /27 | 32 | 30 |  |  |  |
| Department B (Sales): | 28 usable hosts |  |  |  |  |  |
| Allocated Network Address & Prefix Length (/XX) | 192.168.1.64/27 | Even |  |  |  |  |
| ​Subnet Mask | 255.255.255.224 |  |  |  |  |  |
| ​First Usable IP | 192.168.1.65 | Odd |  |  |  |  |
| ​Last Usable IP | 192.168.1.94 | Even |  |  |  |  |
| ​Broadcast Address | 192.168.1.95 | odd |  |  |  |  |
| 192.168.1.96/27 |  |  |  |  |  |  |
| …11110000 | /28 | 16 | 14 |  |  |  |
| Department C (HR): | 12 usable hosts |  |  |  |  |  |
| Allocated Network Address & Prefix Length (/XX) | 192.168.1.96/28 | Even |  |  |  |  |
| ​Subnet Mask | 255.255.255.240 |  |  |  |  |  |
| ​First Usable IP | 192.168.1.97 | Odd |  |  |  |  |
| ​Last Usable IP | 192.168.1.110 | Even |  |  |  |  |
| ​Broadcast Address | 192.168.1.111 | odd |  |  |  |  |
| 192.168.1. 112/28 |  |  |  |  |  |  |
| …11111100 | /30 | 4 | 2 |  |  |  |
| Department D (WAN Link): | 2 usable hosts |  |  |  |  |  |
| Allocated Network Address & Prefix Length (/XX) | 192.168.1.112/30 | Even |  |  |  |  |
| ​Subnet Mask | 255.255.255.252 |  |  |  |  |  |
| ​First Usable IP | 192.168.1.113 | Odd |  |  |  |  |
| ​Last Usable IP | 192.168.1.114 | Even |  |  |  |  |
| ​Broadcast Address | 192.168.1.115 | odd |  |  |  |  |